Electronics and Electrical Communications Engineering
Department

Faculty of Engineering

Cairo University

Submitted to: Dr. Mohsen Rashwan

Speech Processing Problems

DSP-1 Assignment 1 submitted for course ELC4011 “DSP-1 Applications“

4th Year

1st Semester - Academic Year 2025/2026

Prepared by:

| NAME | SECTION | ID |
| --- | --- | --- |
| Yousef Khaled Omar Mahmoud | 4 | 9220984 |

Submission Date: 20 November 2025

# Table of Contents

1	Table of Contents	2

2	Table of Figures	2

3	List of tables	2

DSP-1 Assignment 1: Speech Processing Problems	3

Youssef Khaled Omar Mahmoud*1	3

1	Introduction	3

A.	Audio File Information	3

2	Analyze the Frequency Domain Characteristics of Windows	3

A.	Window Definitions	3

1)	Rectangular Window WRec  [1]	3

2)	Hanning Window WHan  [1]	3

3)	Hamming Window WHam  [1]	3

B.	Time Domain Visualization	4

C.	Frequency Domain Visualization	4

D.	Discussion and Comments	4

1)	Rectangular Window WRec:	5

2)	Hanning Window WHan:	5

3)	Hamming Window WHam:	5

3	Linear Predictive Coding Analysis based on Given Autocorrelation Data	6

E.	Methodology: Solving the Yule-Walker Equations	6

F.	Input Data and Results	6

4	Conclusions	7

5	REFERENCES	7

# Table of Figures

Figure 1 : Time Domain Representation of Rectangular, Hanning, and Hamming Windows (N=1024)[2]	4

Figure 2 : Frequency Domain Analysis of Windows (Log Magnitude)[2]	4

Figure 3 LPC Calculations[3]	6

# List of tables

Table 1 Audio File Information[2]	3

Table 2 Comparison of Frequency Domain Characteristics for Rectangular, Hanning, and Hamming Windows	5

Table 3 Problem 2 LPC Results[2]	7

# DSP-1 Assignment 1: Speech Processing Problems

### Youssef Khaled Omar Mahmoud*1

*Electronics and Communication Department, Faculty of Engineering,

Cairo University Giza, 12613, Egypt

yousef.mahmoud03@eng-st.cu.edu.eg

Abstract: Since

Key words: ).

# Introduction

.ss

## Audio File Information

As shown in Table 1, the analysis in this report, particularly the practical implementation of windowing (Problem 1) and the context for LPC (Problem 2 & 3), is based on the following source audio file:

Table 1 Audio File Information[2]

| Parameter | Value |
| --- | --- |
| File | ..\Data\Test\C02n_1.wav |
| Sampling Frequency Fs | 16000 Hz |
| Bits Per Sample (Bit Depth) | 16 bits |
| Number of Channels | 1 |
| Total Samples | 12800 |
| Calculated Bit Rate | 256000 bits/s |

# Analyze the Frequency Domain Characteristics of Windows

This section presents the time-domain definitions and frequency-domain analysis for the Rectangular, Hanning, and Hamming window functions, which are fundamental tools in digital signal processing for frame-based analysis.

## Window Definitions

Given a window length N, the three specified window functions are defined in the time domain as follows:[1]

### Rectangular Window WRec  [1]

WRecn=1  for 0≤n≤N-10 Otherwise

### Hanning Window WHan  [1]

WHann=0.5-0.5cos2πnN for 0≤n≤N-1

### Hamming Window WHam  [1]

WHamn=0.54-0.46cos2πnN for 0≤n≤N-1

Figure 1 : Time Domain Representation of Rectangular, Hanning, and Hamming Windows (N=1024)[2]

Time Domain Visualization

As shown in Figure 1, the time domain representations of the three window functions for N = 1024 are visualized below. This plot demonstrates how the windows taper the input signal.

## Frequency Domain Visualization

Figure 2 : Frequency Domain Analysis of Windows (Log Magnitude)[2]

The frequency response,Wf, is obtained by computing the Discrete-Time Fourier Transform (DTFT) of the time-domain window, typically approximated using the Fast Fourier Transform (FFT). The plots below show the log-magnitude spectrum, 20log10Wf, normalized such that the maximum peak is 0 dB.

As shown in Figure 2, the plots are centered around 0 Hz and zoomed to the range -Fs/16, s/16 H

Z to clearly observe the main lobe and the first few side lobes. The magnitude floor is clipped at

-80 dB for visualization clarity.

## Discussion and Comments

As shown in Table 2, the primary function of a window is to mitigate spectral leakage by smoothly forcing the signal to zero at the frame boundaries. The analysis of the windows in the frequency domain reveals the critical trade-off between frequency resolution (main lobe width) and spectral leakage (side lobe attenuation).

Table 2 Comparison of Frequency Domain Characteristics for Rectangular, Hanning, and Hamming Windows

| Characteristic | Rectangular WRec | Hanning WHan | Hamming WHam |
| --- | --- | --- | --- |
| Main Lobe Width | Narrowest (Highest Resolution) | Medium | Medium |
| Peak Side Lobe | Highest ≈-13 dB | Moderate ≈-31 dB | Lowest ≈-41 dB |
| Leakage/Attenuation | Worst Spectral Leakage | Good Attenuation | Best First Side Lobe Attenuation |
| Application | Analyzing transient, short bursts | General purpose, good compromise | Better for minimizing interference from strong nearby tones |

Detailed Comments:

### Rectangular Window WRec:

- Main Lobe: Exhibits the narrowest main lobe, theoretically offering the best frequency resolution.

- Side Lobes: Its sudden truncation in the time domain results in the highest side lobes (only ≈13 dB down from the peak). This poor attenuation means energy from a strong frequency component "leaks" significantly into adjacent frequency bins, leading to severe spectral leakage.

### Hanning Window WHan:

- Tapering: Provides a smooth taper to zero, which significantly reduces side lobe levels.

- Side Lobes: The peak side lobe is suppressed to about -31 dB This is a substantial improvement over the rectangular window.

- Trade-off: This suppression comes at the cost of a wider main lobe (approximately twice the width of the rectangular window), which slightly reduces frequency resolution.

### Hamming Window WHam:

- Design: The Hamming window is a modification of the Hanning window, designed specifically to minimize the height of the first side lobe.

- Side Lobes: Achieves the best first side lobe attenuation, suppressed to about -41 dB.

- Trade-off: While the first side lobe is the lowest, subsequent side lobes roll off more slowly than those of the Hanning window.

Conclusion: For applications like speech processing, where minimizing spectral leakage is crucial to accurately separating closely spaced harmonics and formants, the Hanning and Hamming windows are strongly preferred over the Rectangular window. The choice between Hanning and Hamming depends on whether a faster side lobe decay (Hanning) or the lowest possible first side lobe (Hamming) is desired.

# Linear Predictive Coding Analysis based on Given Autocorrelation Data

## Methodology: Solving the Yule-Walker Equations

Problem 2 requires the determination of the Linear Predictive Coding (LPC) coefficients, a1

Figure 3 LPC Calculations [3]

and a2,  and the minimum mean-squared prediction error, E2, given the first three autocorrelation values of a signal, as discussed in the course material on Speech Analysis [3].

As shown in Figure 3, The solution is found by solving the Yule-Walker system of equations, which relates the autocorrelation values R to the LPC coefficients a and the minimum mean-squared error Ep. For a prediction order p, the matrix equation is:

Rpa=rp

where:

- Rp is the p ×p Toeplitz matrix containing autocorrelation values Ri-j.

- a=a1,a2,…,apT is the vector of LPC coef

- rp=R1,R2,…,RpT is the vector of shifted autocorrelation values.

The minimum mean-squared error Ep is calculated as:

Ep=R0+k=1pakRk

## Input Data and Results

The given autocorrelation data for a prediction order p=2 is:

R=R0,R1,R2=1.0000,0.7000,0.4000

Substituting these values into the p=2 Yule-Walker matrix equation:

1.00.70.71.0 a1a2 = 0.70.4

As shown in Table 3, Solving this linear system using the implementation (e.g., using matrix inversion or the Levinson-Durbin algorithm) yielded the following results:

Table 3 Problem 2 LPC Results [2]

| Parameter | Value |
| --- | --- |
| Prediction Order p | 2 |
| LPC Coefficient a1 | 0.8235 |
| LPC Coefficient a2 | -0.1765 |
| Minimum Mean-Squared Error E2 | 1.5059 |

- Conclusions

Due to the suc

#### BIOGRAPHY

Youssef Khaled Omar Mahmoud

Youssef Khaled is an undergraduate student in Electrical and Computer Engineering at the Faculty of Engineering, Cairo University, Egypt. He specializes in embedded systems, IoT, and intelligent control. Youssef has led research and development projects such as AquaVision, an AI-driven smart aquaculture system, and has hands-on experience with STM32, ESP32, C/C++, Python, and sensor–actuator integration. He has also contributed to robotics education and embedded system teams through IEEE CUSB and CUERT.

- REFERENCES

- S. K. Mitra, "Digital Signal Processing: A Computer-Based Approach," 4th ed., McGraw-Hill, 2011.

- "DSP-Speech-Processing Assignment," GitHub Repository. [Online]. Available: https://github.com/youefkh05/DSP_Speech_Processing

- Dr. Mohsen Rashwan, "Lec3&4 Speech Analysis (DSP-1 Applications)," Cairo University, [Lecture Slides], 2025.

المقدمة باللغة العربية